For each point at each curve (measured from the connection or from the lyre side) we calculate the distance from the point linearly interpolated at same Y between the previous and the next points (red segment on the picture below):

This is the code used to build the statistics for all distances:

create table X_COMPLETE_AXIS

as

select distinct meas_id

from CRYOGEO_AXIS

where final_meas = 'T'

group by meas_id, aperture

having count(case when side = 'connection' then side

else null end) > 19

and count(case when side = 'lyre' then side else null end) > 19;

create table X_AXIS_LAG_LEAD as

select A.magnet_nu, A.step, A.meas_id, meas_date, A.aperture, A.side, A.point_id, Y, DX, DZ,

lag(y, 1, null)   over (partition by A.meas_id, A.aperture, A.side order by y) prev_y,

lag(dx, 1, null)  over (partition by A.meas_id, A.aperture, A.side order by y) prev_dx,

lag(dz, 1, null)  over (partition by A.meas_id, A.aperture, A.side order by y) prev_dz,

lead(y, 1, null)  over (partition by A.meas_id, A.aperture, A.side order by y) next_y,

lead(dx, 1, null) over (partition by A.meas_id, A.aperture, A.side order by y) next_dx,

lead(dz, 1, null) over (partition by A.meas_id, A.aperture, A.side order by y) next_dz

FROM CRYOGEO_AXIS A, CRYOGEO_MEAS_ENV B

where A.meas_id = B.meas_id

and side in ('connection','lyre')

and exists (select 1 from X_COMPLETE_AXIS where meas_id = B.meas_id)

order by MEAS_ID, APERTURE, SIDE, Y;

create table X_AXIS_Y_DIST

as

select MAGNET_NU, STEP, MEAS_ID, MEAS_DATE, aperture, side, point_id,

Y, DX, DZ,

DX - pda_utl.interpolateL2D(PREV_Y, PREV_DX, NEXT_Y, NEXT_DX, Y) diff_dx,

DZ - pda_utl.interpolateL2D(PREV_Y, PREV_DZ, NEXT_Y, NEXT_DZ, Y) diff_dz

from X_AXIS_LAG_LEAD

where PREV_Y  is not null

and   PREV_DX is not null

and   PREV_DZ is not null

and   NEXT_Y  is not null

and   NEXT_DX is not null

and   NEXT_DZ is not null;

For all measurements (470013 points in total for 3610 measurements) we have the following statistics:

Average distance   -  0.0007 mm

Standard deviation -  0.0716 mm

Min                        - -1.9548 mm

Max                        -  1.5542 mm

The distribution seems to be close to normal.

Also, most of the big distances are near the tube extremities and cannot be treated as the outliers. The chart below shows the average and the standard deviation of the “smoothness distance” over each 20 cm of the tube, all measurements:

The picture below shows all distances for all last measurements at WP08 for all magnets passed WP09 (ready to be installed in the tunnel).

So the zone with stable distance can be chosen between Y = 1.4m and Y = 13.8m.

The statistics for this zone is the following:

| All measurements: Average = 0.0021mm, standard deviation = 0.0639 mm | Last WP08: average = 0.0037mm, standard deviation = 0.0714 mm |
| --- | --- |

The following query was used to search for the outliers (6σ (0.024% over all points measured) from the mean in the central part of the tube):

select distinct magnet_nu, step, aperture, side

from X_AXIS_Y_DIST A

where y between 1.4 and 13.8

and abs(1000*diff_dz – 0.0021) > 6*0.0639

and (

(step like 'WP08%' and not exists (

select 1 from cryogeo_meas_env

where magnet_nu = A.magnet_nu

and step like 'WP08%' and step > A.step)

)

or step like 'ITP%')

This query gives 60 measurements which are suspicious for the outliers:

| Magnet | Step | Aperture | Side |
| --- | --- | --- | --- |
| 1068 | WP08C-FID | 2 | lyre |
| 1080 | WP08C-FID | 1 | connection |
| 1083 | WP08B-FID | 1 | connection |
| 1088 | WP08B-FID | 2 | lyre |
| 1176 | WP08B-FID | 1 | lyre |
| 1181 | WP08-FID | 2 | lyre |
| 1233 | WP08-FID | 1 | connection |
| 1268 | WP08-FID | 2 | lyre |
| 1271 | WP08-FID | 2 | lyre |
| 1283 | WP08-FID | 1 | connection |
| 1287 | WP08-FID | 2 | lyre |
| 2007 | WP08C-FID | 1 | lyre |
| 2012 | ITP20-GEO | 2 | lyre |
| 2012 | WP08C-FID | 1 | connection |
| 2014 | WP08E-FID | 1 | connection |
| 2014 | WP08E-FID | 1 | lyre |
| 2051 | ITP15-BCA | 2 | lyre |
| 2059 | ITP15 | 1 | connection |
| 2059 | ITP15 | 1 | lyre |
| 2059 | ITP15-BCA | 1 | connection |
| 2059 | ITP20-GEO | 1 | connection |
| 2061 | ITP15 | 2 | connection |
| 2061 | ITP15-BCA | 2 | connection |
| 2086 | WP08B-FID | 2 | lyre |
| 2104 | WP08C-FID | 2 | connection |
| 2108 | ITP15 | 2 | lyre |
| 2108 | ITP15-BCA | 2 | lyre |
| 2109 | ITP15 | 1 | lyre |
| 2109 | ITP15-BCA | 1 | lyre |
| Magnet | Step | Aperture | Side |
| 2113 | WP08B-FID | 1 | connection |
| 2129 | WP08-FID | 2 | connection |
| 2129 | WP08-FID | 2 | lyre |
| 2131 | ITP15 | 2 | lyre |
| 2131 | ITP15-BCA | 2 | lyre |
| 2133 | WP08B-FID | 2 | lyre |
| 2147 | WP08-FID | 2 | connection |
| 2162 | WP08-FID | 2 | connection |
| 2163 | WP08-FID | 2 | lyre |
| 2166 | WP08-FID | 2 | lyre |
| 2179 | WP08-FID | 2 | lyre |
| 2184 | WP08-FID | 2 | lyre |
| 2195 | ITP15 | 2 | connection |
| 2195 | ITP15-BCA | 2 | connection |
| 2203 | WP08-FID | 1 | connection |
| 2204 | WP08-FID | 2 | lyre |
| 2208 | WP08-FID | 1 | connection |
| 2259 | WP08-FID | 2 | lyre |
| 2525 | WP08-FID | 2 | connection |
| 3015 | WP08C-FID | 1 | connection |
| 3173 | WP08B-FID | 2 | lyre |
| 3241 | WP08B-FID | 2 | lyre |
| 3244 | WP08D-FID | 2 | lyre |
| 3263 | WP08-FID | 2 | lyre |
| 3348 | WP08B-FID | 1 | connection |
| 3386 | WP08-FID | 2 | connection |
| 3395 | WP08B-FID | 1 | connection |
| 3412 | WP08-FID | 1 | connection |
| 3413 | WP08-FID | 2 | lyre |
| 3415 | WP08-FID | 2 | lyre |
| 3416 | WP08-FID | 2 | lyre |

Each case should be investigated. A few examples:

Figure 1. Magnet 1283, Ap1.

Figure 2. Magnet 2525, Ap2.

Figure 3. Magnet 2133, Ap2.

Figure 4. Magnet 3412, Ap1.